2018 APBA MODIFIED INSPECTION TEST

This test is divided into two parts:  Inspector Trainees are to answer 1 -  18.  Inspectors are to answer all questions.  You will need a new Tech Manual and a Rule Book.  Please print or type your answers.  You may use additional 8 x 11 sheets of paper if needed.  The completed test should be returned to:  Jeff Williams, Modified Chief Inspector, 2342 Lincoln Way, Ford City, PA  16226 or by email to mjracing@windstream.net  You will be notified of the results.

Name _________________________________ APBA # _______________

Address _______________________________  Phone # _______________

_______________________________  Region ______ Date________

Email address: ________________________________________

True or False for the following:

_____ 1.	Each boat shall be equipped with a spring loaded (closed) throttle and a full carburetor butterfly(s) in all engines.

_____  2.	There is a minimum length for a 500 ccMR Runabout.

_____  3.	The owner shall be responsible for the condition of the motor and hull as raced.

_____  4.	Scales at races cannot be protested, but must be available for use before the first final heat of racing each day.

_____  5.	All life jackets shall have at least 70% of the upper surfaces above the waist, both front and back International Orange or Yellow in color.

_____  6.	You are conducting safety inspections and the driver presents a Snell 2015 helmet for inspection.  The helmet looks pale lime green.  It is legal.

_____  7.	It is permitted to rotate the powerhead on the down housing in all classes.

_____  8.	The Rope Pulley weight of a 102 Yamato in the 500 cc classes is a minimum of 415 gram.

_____  9.	It is permissible to thermal barrier coat piston crowns provided the overall piston crown dimensions are not changed.

_____ 10.	There are no reed cage specifications for the Mercury motor in the 400ccH class.

_____ 11.	It is illegal to coat piston skirts in 350cc Class Yamato 80-M motor.

_____ 12.	The 302 Yamato is a legal 850 cc class motor.  List two reasons why or why not.

A, ___________________________________________________

B.____________________________________________________

_____ 13.	A gear case that measures .950 min S dimension is illegal in the 750 cc classes.

# ENGINE AND BOAT SPECIFICATIONS

14.	200 cc Class

A  _______________	K  ________________	U  _____________

C  _______________	K1 ________________	V  _____________

15.	250 cc class  Hot Rod

G  _______________	J  _________________	R  _____________

S  _______________	Minimum cc’s __________________

16.	Yamato 80  in 350 cc classes

E  _______________	H  ________________	C  _____________

F  _______________	J  ________________	R  _____________

17.	The 25 XS Mercury competing in the 350 ccMR has a propeller shaft height.  What is the dimension? _______

18.	You must measure the “A” dimension in 200 cc classes.  Describe your procedure and tools needed.     ___________________________________________________________

___________________________________________________________

19.	You must measure a Yamato 302 500 cc-class cylinder head to determine legality. How would you accomplish this?  List tools needed or used. ___________________________________________________________

___________________________________________________________

___________________________________________________________

20.     The 850 cc class has finished racing and has reported to Inspection.  The winning boat was powered by a 2 cylinder OMC motor.  The motor has  2 – 2 barrel carbs, is it legal?  _________   Rule ______________________

21.	  You are inspecting a Yamato 102.  You are looking at the cylinders, specifically at the transfer ports.  There are machine marks in the port window of the steel sleeve and these marks extend beyond the sleeve onto the aluminum casting of the cylinder.  Is this legal? ______________

Why, applicable rule(s) _________________________________________

22.	You are measuring a K series carburetor from a Mercury 25 in the 400 cc class.  The throat measures 1.245.  Is this legal? ______________

23.	You are measuring the BV-30 carburetor off a 80-M Yamato.  What should the Venturi measure? _____________

24.  	Please draw a side view of a single reed stop, reed, and portion of the reed cage showing the location where the reed stop height measurement is measured.